# HyperHDR Setup & Boot Fix for CachyOS / Arch Linux

This guide provides a comprehensive manual to download the latest HyperHDR binary release, integrate it directly into your KDE Plasma Application Launcher, and permanently fix the USB serial port (ttyUSB0) detection issue on CachyOS / Arch Linux.

## 🛠️ Section 1: Downloading & Deploying HyperHDR

Since CachyOS runs on Arch Linux, we download the official pre-compiled Linux binary package from the HyperHDR Releases Page.

- Go to the official repository downloads and grab the latest .tar.gz package for Linux architecture (typically x86_64).

- Extract the archive into your home directory under a clean structure:

- mkdir -p ~/HyperHDR
tar -xvf HyperHDR-*.tar.gz -C ~/HyperHDR --strip-components=1

- Note: Ensure the main executable is located exactly at ~/HyperHDR/bin/hyperhdr as shown in your launcher setup.

## 🖥️ Section 2: Adding HyperHDR to KDE Plasma Launcher

To launch HyperHDR elegantly from your applications menu without annoying file manager prompts, create an official desktop application entry.

- Right-click your Application Launcher button (Start Menu) on the Plasma taskbar and select "Edit Applications..." (Editor del menú de KDE).

- Select a target category (e.g., Juegos or Utilidades) and click "+ New / Nuevo" -> "New Item".

- Fill out the General profile precisely matching these fields:

  - Name / Nombre: LedsHDR

  - Program / Programa: /home/YOUR_USER/HyperHDR/bin/hyperhdr (Replace YOUR_USER with your actual Linux username)

- Click "Save / Guardar" in the top right corner.

The application will now appear naturally in your system menu with an assigned icon launcher, bypassing any temporary desktop prompt limits.

## ⚡ Section 3: Automatic USB Ambilight Detection Fix

### The Problem

Hardware serial ports for LED controllers (Arduino, ESP32, CH340 chipsets) register natively under the uucp group on Arch. Systemd boot scheduling triggers a race condition where the USB port sets up boundaries before user profile permissions fully settle inside the active desktop. This causes a detection failure on boot until you either hot-plug the cable or execute manual usermod group updates.

### The Permanent Fix

We bypass rigid group dependencies entirely by applying systemd's dynamic uaccess hardware seat tags. This maps complete communication access directly to whichever user owns the current graphical seat.

#### Step 1: Force Base Group Membership

Map your current user profile directly to the legacy communication grid:

sudo usermod -aG uucp $USER

#### Step 2: Disable Braille Screen Interferences

The native accessibility service (brltty) aggressively hooks USB-Serial communication devices during cold boots, locking HyperHDR out. Permanently disable and mask it:

sudo systemctl stop brltty-udev.service
sudo systemctl mask brltty-udev.service
sudo systemctl stop brltty.service
sudo systemctl disable brltty.service

#### Step 3: Write the Dynamic udev Permissive Rule

Generate a dedicated udev file to catch any serial connection (ttyUSB0, ttyUSB1, etc.), adjust standard boundaries (0666), and tag it for local session management:

sudo echo 'KERNEL=="ttyUSB[0-9]*", SUBSYSTEM=="tty", TAG+="uaccess", MODE="0666"' | sudo tee /etc/udev/rules.d/99-leds.rules

(If your controller chip translates as an ACM device instead, change ttyUSB[0-9]* to ttyACM[0-9]*).

#### Step 4: Refresh System Device Engine

Instruct the running kernel to ingest the new layout adjustments instantly:

sudo udevadm control --reload-rules && sudo udevadm trigger

#### Step 5: Perform a System Reboot

sudo reboot

After restarting your computer, fire up your new LedsHDR shortcut from the Plasma application selection. HyperHDR will capture your ttyUSB0 connection instantly on every boot.